www.reaperbikes.co.nz www.pitsterpro.co.nz www.pitsterpro.com www.thumpstar.co.nz

NEW ZEALAND PITBIKE CLUB, LIKE OUR PAGE www.facebook.com/koarsebikersclub

PDI Sheet Version 4.3
Congratulations on your purchase! You have chosen wisely, the Pit Bike you have bought is one of the best
available. We put every conceivable effort into sourcing & specification in order that you will enjoy your machine to
the full.

This document is supplied only as a guide to a professional and may not be exhaustive. Each machine may require
individual attention & therefore ONLY qualified persons should only carry out the following:

A few tips:

- Choke, most models, down is Choke Off and and up is Choke On. Normally on a warm day choke will not have to be used.

- Kicking over, kicking over needs to be done correctly, failure to do so can cause damage to your bike especially when bike is new and has lots of compression, it needs to kicked over like a 4 stroke not a 2 stroke. To kick over correctly, put bike into neutral then foot on kick start and take slack/play up i.e. when kick start engages into engine, then kick over, this does not need to be done with much force, its only a 125-160 engine not a 500! You may find that sometimes its harder to kick over than others, this is because the Piston is Top Dead Centre(TDC), when this occurs just gently kick it past that point then continue kick it over until it starts.

- Fuel tap, when tap is turned horizontal that means fuel is off and fuel will not leak everywhere. When tap is turned up this normally means its on, some models have reserve so if you run out of fuel you can turn it onto reserve then you will have 1litre spare to get back.

- Rear tyre, most pit bikes do not have rim locks, so ensure your rear tyre has enough air(20-40psi) otherwise you run risk of ripping the valve out, another tip is to have valve nut tight against dust cap NOT the rim. Rim locks can be installed if need be.

- Cleaning, when you clean your pit bike, be careful not to spray to close to decals/stickers and exhaust, put plastic bag over air filter, start bike up after clean and spray with CRC or something similar.

- Last of all, treat your pit bike with respect, keep it clean and do regular checks i.e. following information below and you will have a heap of fun : )

Check List:

Remove Bike & all accessories from crate & inspect to ensure all necessary parts have been supplied

correctly



Mount shock (where applicable)



Mount front wheel & ensure free rotation – check for potential bearing &/or brake drag Note: supermoto

machines – remove calliper before fitting wheel, then replace (use thread lock compound where
necessary)



Ensure all frame bolts are tight (use thread lock compound where necessary)



Loosen triple clamps around fork legs & set desired fork height. Use copper grease on triple clamp bolts to

prevent damage to the alloy threads & then firmly tighten.



Fit handlebars and clutch cable to clutch lever. Note supermoto bikes – when fitting hand guards, it may be

necessary to cut & trim the end of the throttle, you will also need to cut your hand grips. Use copper
grease in alloy threads to prevent damage



Pump front brake lever until firm (bleed if necessary). Check front braking system for potential leaks or

damage by pulling brake lever very hard several times. Visually inspect all joints for leaks & remedy where
necessary. Spin wheel to ensure calliper is releasing from disk



Fit rear brake pedal where necessary & repeat procedure described for front brake



Check fluid levels, top up with DOT4 where necessary



Fit front mudguard using thread locking compound where required.



Fit footpegs



Ensure the correct pressure in tyres 20-40 PSI for off-road, 26 PSI for tarmac track.



Loosen rear wheel & adjust chain tension. Find tightest point in chain by spinning rear wheel, there should 15mm up & 15mm down at the longest section of chain with rider sat on bike. Ensure chain runs true & is
on top of chain roller. Ensure chain does not foul on rear chain guide (use shims/washer to space if
necessary)



Working from front to rear, ensure all nuts & bolts are tight. Where no nyloc nuts or spring washers are

present, ensure thread-locking compound has been used. If none has been used during factory assembly,
remove nut or bolt, apply locking compound & re-tighten. Pay particular attention to the following critical
safety areas: Wheel nuts, brake calliper bolts (thread lock), Top & rear engine bolts (frame must be hard
tight against the engine, use thread lock), chain tensioners, rear shock bolts, triple clamp bolts (yokes),
Handlebars, brake/clutch/throttle, swing-arm, front & rear sprocket



Fully ensure nothing is touching the exhaust system. On oil cooled models, ensure the oil cooler pipes do

not touch the exhaust manifold & that the heat protection springs are located on the correct part of the pipe
in order that should they touch the exhaust the heat will not damage them.



Ensure all cables pipes & wiring are away from moving components such as the wheels, use cable ties

where necessary



Adjust tappets. Set engine to TDC (ONLY rotate engine anticlockwise) & check cam timing alignment

(cam sprocket/cylinder head – adjust is necessary), remove tappet covers & adjust tappet clearances
~0.15mm



TDC for 140/150 engines. Remove flywheel cover & spark plug, find TDC using small

screwdriver or probe & mark rotor in suitable position.



140/150 engines. Remove flywheel cover & set crank position pick-up to fully retarded (as far

anticlockwise as possible). This is as a precaution, to avoid "kick-back" when starting the engine and any
potential engine damage. Fine tuning can be achieved later to your desired ignition timing once carburetion
has been correctly set. Remove both gen coil & pick-up bolts & threadlock. Ensure rotor is tight on end of
crank with no free play. If there is any free-play pack with suitable washers.



Carb – 140 engine / 150 engine – set needle height on second notch from bottom for NZ

conditions. 110 engines normally do not need adjustment



Fuel bike & check for fuel leaks



If fuel leaks from carb overflow – remove carb & adjust float height. Check for any debris which may be
blocking the fuel shut-off valve



Check oil level with bike in upright position using dipstick 10W 40 semi synthetic or mineral oil.



Adjust tension for both clutch & throttle cables.



Ensure wheel spokes are tight & wheels run true.



Start engine & warm up (using choke if necessary). Set idle speed so engine ticks over comfortably
without stalling or racing.



Test machine ensuring that all controls are fluid & functional. Ensure
brakes are working efficiently (bearing in mind that the pads will need to bed in prior to optimum
performance).



Engine break-in

Run bike at no more than 2/3’s throttle & do not allow engine to rev to a high speed. Engine break-in should be no
less than 1 hour.

2 Hour Service and on going services

After 1 hours carry out the following checks & procedures:



Drain oil & replace with a quality semi synthetic 10w 40 4-stroke engine oil



Check all nuts & bolts & tighten where necessary (use PDI data as reference)



Check inner rotor kit (where fitted)



Check wheel spokes (especially rear wheel drive side) & tighten where necessary



Re-Adjust throttle cable & clutch cable if required



Re-tension chain



Check wheel bearings for any sign of wear of free-play



Check brake calliper operation & brake pad wear



Check spark plug gap ~0.6mm-0.7mm



Check tyre pressures

Riding Tips For Long Engine Life - IMPORTANT

Do not over rev your engine as you may cause damage to valves. This is particularly applicable to the 140cc & 150cc
engines, where the inner rotor kit is used. Under load the engine is capable of revving into major valve bounce that
can cause permanent damage to your engine. If you feel the power dropping off or start to hear valve bounce it is
critical that you change up a gear or ease off the throttle.
It is good practise to carry out the above checks on a regular basis, ideally each time you ride your machine. We
advise an oil change after every 10 hours use.
Do not stamp through the gears – always use the clutch.
If you miss a gear do NOT stamp into gear from high engine revs.
Do not drop your clutch heavily, or slip it unnecessarily.

Common Problems:



Exhaust Glows Red (fast tickover) – Carburetion too lean, richen mixture via raising needle



Engine misfires at mid to high engine speed, open throttle – Mixture too rich, drop needle



Engine Pops & bangs on over-run – mixture too rich, drop needle



Engine kicks back hard whilst starting – Ignition too advanced, or mixture too lean



Engine difficult to start – engine flooding or mixture too rich – check float height



Weak Spark – Pick-up coils too far from rotor (where inner rotor kit fitted) or bad earth



No spark – Either poor earth or faulty Rotor Coils/CDI/Coil/Lead/Cap/Plug – use moving coil multimeter to
diagnose



Fuel leaks from overflow – Floats set incorrectly or blocked by debris or sticking



Engine runs fine, but then starts to misfire – faulty rotor coils/CDI/Coil/Plug



Rattles from engine – Tappets set incorrectly



Engine will not tick over when warm – tappets set incorrectly



Engine will not return to idle & races – sticking throttle cable

Correct jetting

Take out the spark plug and check the electrode; the colour should be golden brown. If it is white, the engine is running too lean (not enough fuel). If it is black (sooty colour) it is too rich (too much fuel). If it is white & black, the engine is VERY lean & causing misfire, which you should be able to detect whilst riding. As your jets are close to what they should be for the NZ, minor alterations can be made by adjusting the needle height on your carb, without the need for different sized jets. To add more fuel, raise the needle one notch. This is achieved by removing the carb from the engine, removing the throttle slide, removing the needle & adjusting the position of the circlip. Conversely, if you need to reduce the amount of fuel, drop the needle one notch. Re-assemble the carb & continue testing, remembering to check the colour of your spark plug. Once you have achieved the correct full load fuelling, you can set the idle mixture screw. The 140 & 150 engines like to run very rich at idle, this way you get a clean progression from idle jet to main jet. To archive a richer mixture, turn the air scre (next to filter) clockwise. Idle speed may need to be adjusted at same time to keep engine running.

For more information visit these Forums, if you can not find what you are looking for there give us a Email at targetimports@slingshot.co.nz

www.thumpertalk.com      www.miniriders.com.au      www.pitbikeclub.co.uk